T. CHITWOOD ENGINEERING

TOM CHITWOOD

606 S. PENINSULA DR.

DAYTONA BEACH, FL 32118

PH. 386-290-9982

Steps To Convert 1960 Relay System

1.  Remove the two auxiliary relays and the auxiliary harness from the car (the 2 solenoid relays and their plugs are not going to be used).

2. Remove the 2 solenoid relays and the 4 top and deck control relays marked in yellow on enclosed sheet (sheet 1).

3. Mount the 4 C1SF rebuilt relays provided, in place of the removed C0SF relays (4) and connect to buss bar as before.

4. Reinstall the plugs on the deck and top control relays which will leave one terminal on each relay exposed (see sheet 2).

5. Using the jumper wires provided, connect the 2 exposed terminals on the deck control relays to the blue wire leading into the main harness.

6. As above, use the other jumper wire to connect the terminals on the top control relays to the white wire leading into the main harness.

7. The two plugs and wires removed from the solenoid relays are now inert and may be taped off.

8. If top is stuck in a non-natural position, i.e. top down, tray folded, it must be returned to an operable state before proceeding. If in doubt, call me for help. Note: the tray can be erected or retracted by jumping the appropriate relay.